| Name of activity, event, and location | Sailing (consider in conjunction with Water Activity RA | Date of risk assessment | Feb 2025 | Name of person doing this risk assessment | David Morton Reviewed Ray Hardman |
| --- | --- | --- | --- | --- | --- |
|  |  | Date of next review | Feb 2026 |  |  |

| What hazard have you identified? What are the risks from it? | Who is at risk? | How are the risks already controlled? What extra controls are needed? | What has changed that needs to be thought about and controlled? |
| --- | --- | --- | --- |
| Moving boats around the site for launch and recovery Manual Handling | Staff & Participants | Ensure all rainwater is drained prior to moving Weight of the boat should be assessed and help should be given where needed Before moving boats, participants should be briefed on their role in the move. | . |
| Raising / lowering Masts & Sails Strains /sprains / hand traps | Staff & Participants | Supervision/Help should be given when masts are being placed on the craft or being removed. Ensure mast locking systems are in place Be aware of other people near to the craft, move them clear if too close |  |
| Launch & Recovery of Craft | Staff & Participants | Observe manual handling and safety best practice Briefing to participants / crew |  |
| Attaching/removing rudders/centre boards Finger / hand traps | Participants | Instruct/supervise to ensure safe practice. |  |
| Man Overboard | Participants | Crew briefed in MOB recovery procedure. Safety boat is on hand to provide support and assistance. |  |
| Entrapment | Participants | Safety Boat crew to be aware how many persons are on each craft All capsizes to be attended by a Safety Boat which will have appropriate equipment on board |  |
| Head Injury | Participants | Instruction to include the dangers of the boom crossing the boat Consider use of helmets |  |
| Trapezing | Participants | Possible entrapment following a capsize, Safety Boat to attend |  |